# F/22/93160 – Land adjoining 4 Brookfield, Providence Hill,

# Bursledon, SO31 8AU.

# Context.

This is the second iteration of the third application for this site – the last piece of green lung left on Providence Hill that gave some credence to the rural setting of the Grade ll* listed Bursledon Windmill.  Of the first application I said the following :-

“The proposal is providing new housing with some reference to the local vernacular

in regard to materials, particularly the black weatherboarding, bricks and plain tiles.

There is variety in the building forms and material use .  .  ‘.

Thankfully this application has kept with this and is all the better for it.  Furthermore the change from housing to veterinary practice on the western portion of the site allows a significant portion of the site to be building-free retaining a far more rural feel as one drives up the road and so much more appropriate for the setting of the Bursledon Windmill.

The Proposal.

The western boundary of the ‘agreed easement view to Bursledon Windmill’ appears to have been moved by easing the western boundary to the east when one compares the current plan with the pre-app one, meaning units 1 & 2 and 3 to 5 encroach into the wedge, added to which a row of half a dozen parked cars appear across the space diminishing the remaining rural quality of the view of the setting of the windmill which is of national importance.  The parking for the new housing within the space close to the Providence Hill boundary in the dip, is less obvious.

Units 1 & 2 need to be moved west a bit and units 3 – 5 rotated west a little on the south-east corner in order to follow the wedge boundary and not form a parallel ‘tunnel’ with those on the other side of the road.  The car parking for the vets practice would benefit from some landscaping to break up the apparent extent of it.

The siting of the vets practice building allows it’s bulk to be dissipated by the rising ground and is further ameliorated by stone areas and ‘a front end’ of contrasting materials.  Brick and timber are of course the local indigenous materials and the contrasting stone and curtain walling of the vertical movement element emphasize the public face together with the glazed frontage of the reception area wrapped around the other corner, allowing the animal clients to look out and not seem to be in such an intimidating environment as it would be with normal cill height windows.

The eastern end is the best place to site the plant that will hum, therefore not disturbing residents and it would be best if any a/c units were out of sight.

Conclusion.

This proposal appears to have taken a step back from the point of view of the setting of the Grade ll* listed windmill of national importance.  Holding objection.

18.07.2022. Tim J S Dyer, B.Sc.,Dip.Arch.(Birm), Grad.Dip.Conservation(AA), RIBA. Built Heritage Consultant.