Ecology response updated 18 10 2022 for X/22/93511 Land to the rear of Fairways and Homefield

Variation/ removal of conditions 1, 4, 5, 6, 9 & 10 of planning permission F/21/90597 for the demolition of existing industrial buildings followed by the erection of 7 dwellings with associated parking and use of existing commercial access to serve site. Variation comprises elevational alterations to dwellings and garages.

Clare Martin

18/10/22

I have reviewed the amendments to the CEMP and the site set up plan. I am satisfied that these details will be sufficient.

Lisa Malter

Ecologist

12/9/22

I have reviewed the latest document, the Pollution Mitigation statement by AWA (September 2022) and am satisfied that condition 6 can be discharged now.

Lisa Malter

Ecologist

6/9/22

I have reviewed the Surface Water SuDS Maintenance Manual by AWA (August 2022). I understand that the recommendation of salt for de-icing (2.3 in table 2, page 7) is to avoid having silt and sediment enter the pores of the permeable/pervious paving. However, the salty water will end up in the ponds and be detrimental to the growth of the aquatic vegetation designed to provide additional treatment to the runoff. Salt would be a pollutant in this system and would be carried in solution into the existing watercourse downstream of the site. The pollution hazard levels from the SuDS manual and the mitigation levels for each of the proposed treatments are not included in this document and or in the drainage documents for the original application F/21/90597.

Lisa Malter

Ecologist

1/9/22

Conditions 4 and 9

I have reviewed the submitted drainage information but what I really need is a surface water drainage strategy document with a written explanation of how this system is proposed to work and specific information on the proposed 3 stages of filtration for the surface water. This should include the pollution hazard levels from the SuDS manual and the mitigation levels for each of the proposed treatments. I can see the components which have been proposed but no information on how they are supposed to work together.

The cover letter mentions a Surface Water Drainage Management and Maintenance Plan and the CEMP but I don't see these in the Salesforce documents.

Lisa Malter

Ecologist