Ecology response for F/22/93934 Area of green land on Steele Close, Chandlers Ford, Eastleigh, SO53 3AA

The works include the creation of a temporary site compound area during the replacement of Weardale Road bridge. The compound shall comprise: containers for site offices and welfare, toilet facilities, car parking area for approx. 7 vehicles, area for storage of plant/equipment/materials.

Alex Webb

8/11/22

From the photo of the site on Google maps, this appears to be a closely mown sward of amenity grassland with some bare ground on the northern edge. There are records of slow worms along the railway corridor to the south of the M3. If this site has not developed any tussocky grassland, scrub, or other suitable reptile habitat, then I have no ecological objections. The site condition must be confirmed with a current photo and any grass on site must be kept closely mown to prevent suitable reptile habitat from developing.

Biodiversity enhancement will be required This could include planting areas of flowering plants or shrubs to attract pollinators and mounting at least one bat or bird box on a suitable tree. After the temporary site compound is removed, the site must be restored to an agreed standard.

Any lighting must be directed away from trees and boundary vegetation.

Lisa Malter

Ecologist