H/22/94423 – 148 Church Road, Bishopstoke, SO50 6DS.

Context.

This heritage asset is situated at the top of Church Road well away from the northern most tip of the Bishopstoke Conservation Area within a row of other dwellings.  The nearest other listed building is Stoke Lodge down a long drive on the other side of the road.

This building has suffered some indignities over the years, probably not all with consent, but the most important aspect the front elevation, has at least escaped relatively untouched.

The Proposal.

The scheme will tidy up the existing building and the extension to the rear mask the entirely inappropriate ground floor apertures that have appeared over the years.  The flash gap of the new extension adjacent to the existing building, while shown dark as requested is not annotated as to the treatment which should be added to the drawing.  The back surface only of this requires a dark treatment.

The removal of the slate hanging to the rear and side elevations, added with the best intentions to reduce water penetration no doubt, will enhance the appearance of the building but sounds as if it may well reveal some other nasties to be corrected.  Replacement of the front porch will restore the frontal appearance and enhance its presence no end to give it the status it deserves once again.

Conclusion.

Provided a note is added to the drawing regarding the dark treatment to the back of the flash-gap between old and new at the rear , no objection.

19.01.2023.

Tim J S Dyer, B.Sc.,Dip.Arch.(Birm), Grad.Dip.Conservation(AA), RIBA.

Built Heritage Consultant.