From: Scott, Neil <Neil.Scott@eastleigh.gov.uk> 
Sent: Tuesday, October 17, 2023 1:26 PM
To: Martin, Clare <Clare.Martin@eastleigh.gov.uk>
Subject: Pollution comment - F/23/96022 25 HIGH STREET, BOTLEY: Construction of 1no. two storey detached dwelling and garage within rear garden

Clare

Thank you for consulting us on the above application, with respect to air and noise pollution we have the following comments (any comments on land contamination will be forwarded separately):

Noise:

The proposed dwelling is located off Botley High Street, which is a well trafficked through route, and adjacent to commercial uses, tyre repair and MOT workshop with vehicle parking, which have the potential to impact the development. In this respect the application is supported by a Noise Impact Assessment report, by AIRTIGHT & NOISECHECK LIMITED, Job No. 1446, dated 23.5.2023. The report advises that the development can proceed on the basis that the proposed noise mitigation measures are implemented and that on this basis there is ‘…no reason to suggest that the future residents of the proposed residential development will be adversely affected...’.

Whilst we do not disagree with this conclusion in respect of road traffic noise, we have concerns regarding the suitability of the assessment of the adjacent commercial noise sources. Typically, the assessment of commercial noise impacts in such situations as this is by reference to British Standard BS4142 (BS 4142:2014+A1:2019 Methods for rating and assessing industrial and commercial sound). The report makes only a passing reference to the adjacent commercial use and a detailed assessment has not been included. The assessment should also consider the permitted operational hours on the adjacent land, as well as the other uses which could legitimately operate from the neighbouring land within the existing use class / permission. The report also appears to suggest that the property will not have noise sensitive rooms on the eastern façade however the plans show bedrooms overlooking the commercial use.

In terms of the acoustic glazing specified we are not clear whether free field or façade levels have been used in calculations, however, until the matter of the noise assessment of the commercial uses has been addressed the matter is moot. Reference is also made to having optimised the layout of the development site with respect to noise impacts, however the dwelling is located with two sides adjacent to the commercial land. From the information provided it is not apparent how this optimises the layout in acoustic design terms, where frequently the approach is to separate noise making and noise sensitive uses.

Air Pollution:

The development would increase the number of vehicles traveling along the Botley High Street, Air Quality Management Area, on this basis we would recommend that provision is made on the development to facilitate electric vehicle charging.

With respect to the neighbouring commercial use information is not provided on the potential or likelihood of odour, smoke or other emissions to air impacting the development.

During development there is potential for the construction works to impact on nearby dwellings, we would therefore advise that conditions are applied with respect to: hours of work, no burning on site, and appropriate measures to be taken to manage noise and dust impacts.

Land Contamination:

Advice will be provided separately.

At this time we do not support the application as insufficient has been provided to demonstrate that significant adverse noise impacts from adjacent commercial uses have avoided, and adverse impacts otherwise minimised.

Regards

Neil

Neil; Scott.. Senior Pollution Control Officer. Service Delivery - Specialist Services - Environment. Ext.:; 8304... Mobile: 07467012360..

Banner springclean​

# From: Planning (Eastleigh Borough Council) <planning@eastleigh.gov.uk> 
Sent: Tuesday, September 19, 2023 3:40 PM
To: Pollution.CMO <Pollution.CMO@eastleigh.gov.uk>
Subject: Planning App Consultation Request Internal F/23/96022

Application No: F/23/96022
Address: 25 HIGH STREET, BOTLEY, SOUTHAMPTON, SO30 2EA
Description: Construction of 1no. two storey detached dwelling and garage within rear garden

Would you please let me have your observations on the above application.
 
You can access the plans and documents through Salesforce. At the top of the screen type the planning application number into the search box and either select the application from the list displayed or press the enter key and select it from the list of results.  

In order to meet the Government’s challenging targets for our speed of determining applications, we require any comments you wish to make within 14 days from the date of this memo.  Failure to meet this time period will usually result in an application being determined without your comments being considered.
 
Please reply to the planning officer on the below email address.
 
Planning Specialist Contact Details
Email: clare.martin@eastleigh.gov.uk
Phone/Ext:  023 8068 8256
 
Kind regards
Planning