Memorandum

| From: Pollution Control | To: Development Management |
| --- | --- |
| Author: Yacklin Panza | FAO: Summer Sharpe |
| Our Ref: PR202311-102207 | Your Ref: F/23/96407 |

Date: 22 January 2024

Application No: F/23/96407

Address: BURSLEDON HALL, BLUNDELL LANE, BURSLEDON, SOUTHAMPTON, SO31 1AA

Description: Conversion of existing office buildings at Bursledon Hall to create 16no. residential flats.

Thank you for consulting this Service on the above application. We have reviewed the information provided and note the following:

Contaminated land

The site is not within an identified contaminative area; however, made ground is anticipated due to the historical development of the site (as Children’s Hospital from 1959 until 1980s, and currently as Research building). There are several surface water features in the vicinity of the site, which are Hoe Moor Creek and River Hamble located at approximately 550m to the southeast and a pond located at 200m northwest. The site is not within a radon risk area. The proposal development does not include the demolition of any existing structures. It is not clear if new landscaping is proposed.

Previous application PN/21/91216 recommended that standard contaminated land conditions (desk study, site investigation, remedial scheme, verification report and unexpected contamination) should be imposed prior development. We support the previous conditions recommended, and would like to add the following:

CEMP

() No development shall start on site until a Construction Environmental Management Plan (CEMP) has been submitted to and approved in writing by the Local Planning Authority, which shall include: a) Methods of construction works; b) The arrangements for deliveries associated with all construction works; c) Location of temporary site buildings, compounds, construction material and plant storage areas, waste disposal areas, etc; d) Measures to minimise dust, noise and vibration; e) Measures to ensure dirt is not transferred onto the highway from the site; f) Measures to ensure that nearby surface water and existing landscape features are protected during the development. Construction work shall only take place in accordance with the approved CEMP.

Reason: In the interest of amenity in accordance with Policies DM1 and DM8 of the Eastleigh Borough Local Plan (2016- 2036).

Imported soils

() Any imported soils required for the development hereby permitted should be demonstrated to be suitable for its intended end use and meet the approval of the Local Planning Authority prior to use on site.

Reason: To minimise the risk from land contamination for the safety of the future occupiers of the development.

Noise

The site is affected by noise from the M27; an assessment should be carried out to demonstrate that actions are being taken to minimize the impact of noise to the new proposed flats.

Internal layout does not seem to be optimal (i.e. sensitive areas such as bedrooms located next to communal areas). The applicant should demonstrate good acoustic design has been used to minimise noise (and vibration) impacts, rather than relying on acoustic insulation alone to mitigate noise impacts.

In addition, we would like to know more details about cooling/ heating systems, i.e. will the building have a communal boiler, or will each flat have individual boilers, location of boilers, flue emissions, etc.

Air quality

The site is located approximately 0.5km from the Hamble Lane AQMA.

Upon the initial review, it was noted that several diffusion tubes monitoring locations within the vicinity have the potential to exceed or come close to exceeding the annual air quality objective (before bias and distance adjustment). Given the proximity to the AQMA, we recognise the importance of a comprehensive assessment of the potential impacts on air quality.

As the proposed development falls under the category of "major development" according to The Town and Country Planning (Development Management Procedure) (England) Order 2015 (involving more than 10 dwellings), it is required to conduct an Air Quality Assessment (AQA) to evaluate the potential impact of such development on local air quality, especially considering the existing challenges in the nearby AQMA.

We recommend that the Air Quality Assessment addresses the following key aspects such as baseline review, assessment, and mitigation measures.

We stress the importance of adopting best practices and measures to safeguard air quality in the area. The planning process should ensure that the proposed development aligns with air quality objectives, promoting a healthy and sustainable environment.

Should you wish to discuss the matters raised here I shall of course be pleased to do so.

Yours Sincerely,

Yacklin Panza

Pollution Control Specialist