Flood risk statement

This statement is to identify the flood risk of the additional rainwater and the height of the DPC level of the proposed side extension that is being proposed at 10 Monks Brook Close Eastleigh.

The proposed side extension is being built in replace of a lean to. The extension will extend to the point of the existing dwelling but go no further.

The new roof to the side extension will be a gable roof design with a continuous gutter along the rear elevation. The other half of the pitched roof will run to a gully gutter between the new roof and the existing garage roof. There is a small increase in the size of the roof due to the extension but existing soak aways will be used.

The DPC level of the new extension will match that of the existing structure and therefore the flood risk stays the same. The DPC is 500mm off the ground to the rear of the extension.

I have attached the flood risk map on a separate document obtained from the government website and would like to highlight that the property itself for flooding is within the low-risk area. Most of the garden is either within flood risk zone 2 or 3, however the extension would not extend out that far out and it is at the top end of the garden. Other properties have had extensions , garages and sheds that and are all closer to the flood zone than this proposal.

In terms of surface water risk again predominantly the garden is within the low-risk area and so the precautions of the DPC level maintaining the same height as the existing DPC should suffice. There is also potential to store rainwater in a water butt as well.

surface water – Low risk